Definitions :
(i) Relevant Failure : A relevant failure of an item is an independent failure which results in a loss of function of that item caused by any of the following:

- A fault in an equipment or sub-system while operating within its design and environmental specification limits;

- Improper operation, maintenance, or testing of the item as a result of the Contractor supplied documentation.

- Failures of transient nature including those with post investigation status as ̳No fault found‘, shall be considered as relevant failure if in the opinion of the Engineer theseare attributable to rolling stock. The decision of the Engineer shall be final.

(ii)  Non-relevant Failure: Any failure of an item not included in the definition of relevant failure, such as the following:

- A failure caused by malfunction of other equipment or subsystem that are not supplied by the Contractor;

- A failure caused by human error, except as noted in Relevant Failure above;

- A failure caused by accidents not associated with the normal operation of the item. Such as collision or striking a foreign object on the right of way;

- A failure caused by operating the equipment or sub- system outside of design or environmental specification limits.

(iii) Service Failure:  Any relevant failure or combination of relevant failures during revenue service operations, simulated revenue operations, or during pre-departure equipment status checkouts to determine availability for revenue service, which results in one of the following:

- Non-availability of the train to start revenue service after successful completion of pre-departure checkout;

  - Withdrawal of the train from revenue services;

  - A delay equivalent to or exceeding 3 minutes.

The discretion of declaring a metro train as Not-available to start revenue service after successful completion of pre- departure checkout or withdrawing a train from revenue service on account of any relevant failure rests solely with the Employer and shall be final.

(iv) Pattern Failure: Repeated occurrence of three or more relevant failures of the same replaceable part, item or equipment in same manner in identical or equivalent applications when they occur at a rate, which is inconsistent with the predicted failure rate of the part, item or equipment. The percentage predicted rate shall be as under:

(a) three or more relevant Service Failures of the same replaceable part, item or equipment in same manner in identical or equivalent applications occur at a rate which is at least 20 % higher than the predicted failure rate of the part, item or equipment and / or

(b) at least 20 % of the same replaceable part, item or equipment in the fleet has a relevant failure in the same manner in identical or equivalent applications during a moving 18 months window starting when the reliability demonstration starts and ending at the end of the Defect Liability Period, these failures will be classed as pattern failures.

(v)Mean Distance Between Failure (MDBF): The MDBF is the ratio of the total operating distance accumulated by the total available fleet of the trains to the total number of Service Failures

(vi) Mean Distance Between Component Failures (MDBCF): The MDBCF of a system is the ratio of the total operating distance accumulated by the total population of